DESIGN AXI-APB BRIDGE BASED ON AXI 4.0 AND APB 4.0 PROTOCOL

ABSTRACT

With the rapid development of technology in deep Submicron, the scale of integrated circuit (IC) grows increasingly. Intellectual property (IP) reuse has been taken popularly. This design method is widely applied in the system on chip (SoC) field. The on-chip bus design is most cared in IP-reused based SoC design. Several bus standards were proposed. Among those standards, the advanced microcontroller bus architecture (AMBA) is the most favored. ARM introduced the Advanced Microcontroller
Bus Architecture (AMBA) 4.0 specifications in March 2010, which includes Advanced eXtensible Interface (AXI) 4.0. This is very high-speed bus. AMBA bus protocol has
become the de facto standard SoC bus. That means more and more existing IPs must be able to communicate with AMBA 4.0 bus, we designed an Intellectual Property (IP) core of Advanced Peripheral Bus (APB) bridge, which translates the AXI4.0 transactions into APB 4.0 transactions. The bridge provides an interface between the high-performance AXI bus and low-power APB domain.

I.INTRODUCTION

Integrated circuits have entered the era of System-on-Chip (SoC), which refers to integrating all components of a computer or other electronic system into a single chip. It may contain digital, analog, mixed-signal, and often radio-frequency functions – all on a single chip substrate. With the increasing design size, IP is an inevitable choice for SoC design. And the widespread use of all kinds of IPs has changed the nature of the design flow, making On-Chip Buses (OCB) essential to the design. Of all OCBs existing in the market, the AMBA bus system is widely used as the de facto standard SoC bus.

On March 8, 2010, ARM announced availability of the AMBA 4.0 specifications. As the de facto standard SoC bus, AMBA bus is widely used in the high-performance SoC
designs. The AMBA specification defines on-chip communication standard for designing high-performance embedded microcontrollers. The AMBA 4.0 specification defines five buses/interfaces:

- Advanced eXtensible Interface (AXI)

- Advanced High-performance Bus (AHB)

- Advanced System Bus (ASB)

- Advanced Peripheral Bus (APB)

- Advanced Trace Bus (ATB)

AXI, the next generation of AMBA interface defined in the AMBA 4.0 specification, is targeted at high performance, high clock frequency system designs and includes features which make it very suitable for high-speed sub-micrometer interconnect.

- separate address/control and data phases

- support for unaligned data transfers using byte strobes

- burst based transactions with only start address issued

- issuing of multiple outstanding addresses

- easy addition of register stages to provide timing closure

II. PROPOSDE HARDWARE ARCHITECTURE OF AXI APB BRIDGE

II.1. APB PROTOCOL

APB belongs to the AMBA family and applicable for a single master and slave principle. It is optimized for low power, reduced complexity interface and low bandwidth operations which is unpipelined protocol. It is supported only for a single master and multiple slaves and contains only single channel to communicate between master and slave usually done by PSELx signal. PSELx signal is a master signal which is used to decide the number of slaves are used, PWRITE is a signal from master which decides Read or write operation if PWRITE =1, for write operation and 0 for Read operation. For any operation, PADDR and PWDATA are address and data signals set on the bus as well as PSEL is high, PENABLE must be high in the next clock cycle, when PREADY which is from Slave is high indicating data transfer is completed. PSLVERR which is a slave signal indicates an error condition on Read and Write operations. The transaction of APB is illustrated by the state machine and is shown in the Figure 1.

Figure 1: APB state machine for one transfer

II.2 AXI PROTOCOL

AXI4 refers to advanced extensible peripheral bus it supports for out of order response, multiple outstanding transactions, burst operations and disjoint operations along with advanced peripheral bus. It consists of five independent channels namely Address Write, Write Data, Address Read, Read and Write Response channel. The signal description of each channel is shown in Table 1.

Table 1: Signals of each AXI channel

| Write Address Channel | Write Data Channel | Write Response Channel | Read Address Channel | Read Data Channel |
| --- | --- | --- | --- | --- |
| AWVALID | WVALID | BVALID | ARVALID | RVALID |
| AWREADY | WREADY | BREADY | ARREADY | RREADY |
| AWID[7:0] | WDATA[31:0] | BID[7:0] | ARID[7:0] | RDATA[31:0] |
| AWADDR[31:0] | WSTRB[3:0] | BRESP[1:0] | ARADDR[31:0] | RID[7:0] |
| AWLEN[7:0] | WLAST |  | ARLEN[7:0] | RRESP[1:0] |
| AWSIZE[2:0] |  |  | ARSIZE[2:0] | RLAST |
| AWBURST[1:0] |  |  | ARBURST[1:0] |  |
| AWPROT[2:0] |  |  | ARPROT[2:0] |  |

Write transaction:

In the write transaction, address signal AWADDR and WDATA signal is set on the channel from the master and AWVALID signal which is master to slave goes high, and after one clock cycle a signal AWREADY from slave to master goes to high. Whenever a valid data is transmitted WVALID signal from master to slave goes high and after one clock cycle WREADY signal which is a slave to master signal goes high indicating that a successful transaction is completed. BRESP which is a signal from slave to master is set to OKAY after completion of data transfer. BVALID signal becomes high indicating successful transferring of data, and BREADY is a master to slave signal is high throughout the transaction. The architecture of write transaction is as shown in the Figure 2.

Figure 2: Write Transaction Diagram

Read Transaction:

In Read Transaction, read address signal ARADDR is set on channel, ARVALID signal which is master to slave becomes high, and in the next clock cycle ARREADY which is a master to slave signal becomes high indicating a valid address, and rad channel data signal RDATA which data presents that is to be read by master and RREADY is slave to master signal becomes high after completion of each transaction, RVALID is high which is a master to slave signal indicating successful read transaction, RLAST is a is high for last read burst , the mechanism for read transaction is as shown in the below Figure 3.

Figure 3: Read Transaction Diagram

II.3 PROPOSED HARDWARE ARCHITECTURE

In this section, we present the proposed hardware architecture of AXI2APB bridge that includes read request fifo (AR_SFIFO), read data fifo (RD_SFIFO), write request fifo (AW_SFIFO), write data fifo (WD_SFIFO), Decoder, Arbiter, Slave Addr Register and two protocol interfaces that are AXI slave interface and APB master interface. The architecture is described in Figure 4. Before diving into each component in detail, we outline several choices of our design.

The choices of design:

- Only supports 4-byte transfer

- Supports unaligned address

- Synchronous clock

- Supports 4 APB slaves (Table 2)

Table 2: Address Range of 16 APB slaves

| APB Slave | Base Address | End Address |
| --- | --- | --- |
| SLAVE0 | 32’h0000_1000 | 32’h0000_1FFF |
| SLAVE1 | 32’h0000_2000 | 32’h0000_2FFF |
| SLAVE2 | 32’h0000_3000 | 32’h0000_3FFF |
| SLAVE3 | 32’h0000_4000 | 32’h0000_4FFF |

Figure 4: Proposed hardware architecture of AXI2APB Bridge

AR_SFIFO: this fifo stores information of read request channel that consists of ARADDR, ARID, ARLEN, ARSIZE, ARBURST and ARPROT; therefore, it also takes responsibility for handling ARVALID – ARREADY handshake.

AW_SFIFO: this fifo stores information of write request channel that consists of AWADDR, AWID, AWLEN, AWSIZE, AWBURST and AWPROT; therefore, it also takes responsibility for handling AWVALID – AWREADY handshake.

RD_SFIFO: this fifo stores information of read data channel that consists of RDATA, RID, RLAST, RRESP; therefore, it also takes responsibility for handling RVALID – RREADY handshake.

WD_SFIFO: this fifo stores information of write data channel that consists of WDATA, WSTRB, WLAST; therefore, it also takes responsibility for handling WVALID – WREADY handshake.

BRSP_SFIFO: this fifo store information of write response channel that consists of BRESP, BID; thus, it also is responsible for handling BVALID-BREADY handshake.

Note: all mentioned FIFOs are First-word fall-through (FWFT) FIFOs in our design

ARBITER: AXI protocol has pipeline operation and APB protocol has non-pipeline transferring, so we have to handle arbiter mechanics to ensure that there must be one request to be executed at any moment and it can be write transaction or read transaction. This block takes a role of it, described in figure 5 and figure 6.

This block has operation principle as follows:

- After reset, read request is prioritized

- On completion of any request or no transaction is progressing, there are such two cases:

- As write is granting, if read request is waiting, arbiter block prioritizes read in the next times otherwise write is continuously granted.

- As read is granting, if write request is waiting, arbiter block prioritizes write in the next times otherwise read is continuously granted.

Figure 5: Block diagram of Arbiter block

Figure 5 shows the main signals of Arbiter block and their descriptions are discussed as follows:

- aclk, aresetn: AXI clock and low-level reset

- burst_done: informs the completion of current AXI transacwrition as it asserts

- sfifo_ar_empty: shows the empty status of AR_SFIFO(Figure 4) as it asserts

- sfifo_aw_empty: shows the empty status of AW_SFIFO(Figure 4) as it asserts

- write address and control: AWADDR, AWSIZE, AWLEN, AWBURST

- read address and control: ARADDR, ARSIZE, ARLEN, ARBURST

- trans_addr: address of each transfer

- next_trans_rdy: address and data of next transfer are available as it asserts

Besides, this submodule has 2-bit register abt_grant[1:0] that is accountable for doing arbitration between read and write request. Operation of this register is described in Figure 6.

- abt_grant[0] is equivalent to read is whether or not granted

- abt_grant[1] is equivalent to write is whether or not granted

Figure 6: Logic Diagram of Arbitration Operation

Figure 6 shows the three main functions of arbitration operation:

      - White color part illustrates the high priority to read after reset with value of abt_grant register is 2’b01 – abt_grant[0] represents read and abt_grant[1] controls write.

- Gray color region presents control signal that takes a role of updating value of abt_grant Register during run-time process. In addition, burst_complete informs current transaction done as being high logic while burst_going has high logic that is equivalent to one transaction (read or write) is occurring.

- Green color domain describes the handling of arbiter logic that ensures only read or write to be implemented in any time.

DECODER: this block has block diagram described in Figure 7 and takes two main roles as follows:

- Decode ARADDR and AWADDR to determine which APB slave that master accesses to.

- Detect error of address

Figure 7: Block diagram of Decoder component

Figure 7 shows main signals of Decoder block and their descriptions are discussed as follows:

- start_addr_i: start address of APB slave that master want to access

- disallowed_trans_i: unsupported transaction of this design: not 4-byte transfer, unaligned address of WRAP or INCR burst

- dec_error_o: start address is outside valid range of address that design support

- true_psel_o: this signal is equivalent to PSEL in APB interface

COUNTER: this block has block diagram described in Figure 8 and takes two main roles as follows:

Figure 8: Block diagram of COUNTER submodule

Figure 8 shows main signals of Counter block and their descriptions are discussed as follows:

- pclk, preset_n: clock and active-low reset of APB interface

- len_of_burst_i: length of current transaction

- beat_cnt_incr_i: request to increase counter one unit as it is HIGH

- burst_done_o: this signal is HIGH as current transaction is completely done