|  | X2P register | Test case name |
| --- | --- | --- |
| Statements | 100% | Reset case |
| Branches | 100% |  |
| FEC Expressions | 75% Reason: - FEC Expression: errorCondition=(psel & pwrite) ¦ (paddr[1:0] != 2'b00) ¦ (paddr[31:0] > 32'h0000_0004) - This element (paddr[1:0] != 0) is not active |  |
| FEC Conditions | 100% (psel & ~penable) |  |
| Toggles | 94.4% Reason: prdata[16:13] |  |
|  | X2P core |  |
| Statements | 99.49 % Reason: Non-executed lines: - 369: (?) - 647: (current test case always ensures new request available after completion of one request => this line never runs => proposed test case: On completion of one transaction, nothing is waiting.) ??? - 816: RESERVED burst |  |
| Branches (if-else) | 95.4% Reason: Non-executed parts: - Default case of currentState comb block - Default case of burstMode (RESERVED) |  |
| FEC Expressions (check the logic optimization of expressions) | 79.71% Reason: Non-executed logic: - assign sfifoArRe = sfifoArNotEmpty & transCompleted & abtGrant[0]; - This signal (sfifoArNotEmpty does not hit “0” logic => the signal is always high => to be considered removing - assign transCntEn = (¦psel[`SLAVE_CNT-1:0] ¦ falseReg ¦ pselRegBlock) & penable & preadyX; - psel, falseReg, pselRegBlock do not hit “0” logic => those signals are always high => to be considered removing - assign sfifoRdWe = sfifoRdNotFull & transCntEn & ~pwrite & abtGrant[0]; - sfifoRdNotFull does not hit “0” logic => the signal is always high => to be considered removing - assign sfifoAwRe = sfifoAwNotEmpty & transCompleted & abtGrant[1]; - sfifoAwNotEmpty does not hit “0” logic => this signal be always high => to be considered removing - assign update = ((abtGrant[0] ? (sfifoAwNotEmpty & sfifoArEmpty) : (sfifoArNotEmpty & sfifoAwEmpty)) & ~transfer) ¦ transCompleted; - sfifoArNotEmpty: '0' hit but '_1' is not hit - sfifoAwEmpty: '0' hit but '_1' is not hit - transfer: '_0' hit but '_1' is not hit - proposed revised version: assign update = (transfer ? 1'b0 : (abtGrant[0] ? (sfifoAwNotEmpty & sfifoArEmpty) : (sfifoArNotEmpty & sfifoAwEmpty))) ¦ transCompleted; - assign preadyX = ¦preadyOut[`SLAVE_CNT-1:0] ¦ falseReg ¦ pselRegBlock & preadyReg; - preadyReg: '_0' not hit - assign pslverrX = ¦pslverrOut[`SLAVE_CNT-1:0]¦ falseReg ¦ pselRegBlock & pslverrReg ¦ (burstMode == 2'b11) ¦ (bytes_in_transfer != 8'd4); - (burstMode == 2'b11): '1' not hit - assign transEn = (¦sel[`SLAVE_CNT-1:0] ¦ false_startAddr ¦ selRegBlock) & (abtGrant[0] ? sfifoArNotEmpty : sfifoAwNotEmpty) & new_req_rdy; - sel: ‘1' hit but '_0' is not hit - false_startAddr:‘1' hit but '_0' is not hit - selRegBlock: ‘1' hit but '_0' is not hit - those signals are always high => to be considered removing |  |
| FEC Conditions | 100% |  |
| Toggles | 98.16% Reason: - bit3Addr[1:0] => 4-byte aligned address - bit4Addr[0] => - sfifoWdWstrb[3] |  |
|  | X2P register block | write_read_write test |
|  | X2P core |  |
| FEC expressions | - assign update = ((abtGrant[0] ? (sfifoAwNotEmpty & sfifoArEmpty) : (sfifoArNotEmpty & sfifoAwEmpty)) & ~transfer) ¦ transCompleted; - transfer: '0' hit but '_1' is not hit => waveform shows emergence “0” and “1” of transfer signal leads to logic ‘1’ never has impact on the expression (cause expression true or false) => Through deep analysis, I guess that while transfer asserts, previous logic (orange logic) is false leading to compiler does not examine the logic of transfer => ‘1’ is not hit => must revise expression to have better version (*) - proposed revised version: (*) assign update = (transfer ? 1'b0 : (abtGrant[0] ? (sfifoAwNotEmpty & sfifoArEmpty) : (sfifoArNotEmpty & sfifoAwEmpty))) ¦ transCompleted; |  |